PXELINUX

A bootloader for Linux using the PXE network booting protocol

Copyright (C) 1994-2001 H. Peter Anvin

This program is provided under the terms of the GNU General Public

License, version 2 or, at your option, any later version.  There is no

warranty, neither expressed nor implied, to the function of this

program.  Please see the included file COPYING for details.

----------------------------------------------------------------------

PXELINUX is a SYSLINUX derivative, for booting Linux off a network

server, using a network ROM conforming to the Intel PXE (Pre-Execution

Environment) specification.  PXELINUX is *not* a program that is

intended to be flashed or burned into a PROM on the network card; if

you want that, check out NILO (http://www.nilo.org/).  NILO can also

be used to create a PXE-compliant boot PROM for most network cards

which have drivers for Linux or FreeBSD.

++++ HOW TO CONFIGURE PXELINUX ++++

PXELINUX operates in many ways like SYSLINUX.  If you are not familiar

with SYSLINUX, read syslinux.doc first, since this documentation only

explains the differences.

On the TFTP server, create the directory "/tftpboot", and copy the

following files to it:

pxelinux.0		- from the SYSLINUX distribution

any kernel or initrd images you want to boot

Finally, create the directory "/tftpboot/pxelinux.cfg".  The

configuration file (equivalent of syslinux.cfg -- see syslinux.doc for

the options here) will live in this directory.  Because more than one

system may be booted from the same server, the configuration file name

depends on the IP address of the booting machine.  PXELINUX will

search for its config file on the boot server in the following way:

First, it will search for the config file using its own IP address

in upper case hexadecimal, e.g. 192.0.2.91 -> C000025B

(you can use the included progam "gethostip" to compute the

hexadecimal IP address for any host.)

If that file is not found, it will remove one hex digit and try

again.  Ultimately, it will try looking for a file named "default"

(in lower case).

As an example, for 192.0.2.91, it will try C000025B, C000025,

C00002, C0000, C000, C00, C0, C, and default, in that order.

It should be noted that all filename references are relative to the

directory pxelinux.0 lives in (usually /tftpboot).  PXELINUX

generally requires that filenames (including any relative path) are 31

characters or shorter in length.

PXELINUX does not support MTFTP, and I have no immediate plans of

doing so.  It is of course possible to use MTFTP for the initial boot,

if you have such a setup.  MTFTP server setup is beyond the scope of

this document.

++++ SOME NOTES ON THE TFTP SERVER ++++

PXELINUX currently requires that the boot server has a TFTP server

which supports the "tsize" TFTP option (RFC 1784/RFC 2349).  The

"tftp-hpa" TFTP server, which support options, is available at:

http://www.kernel.org/pub/software/network/tftp/

ftp://www.kernel.org/pub/software/network/tftp/

... and on any kernel.org mirror (see http://www.kernel.org/mirrors/).

Another TFTP server which supports this is atftp by Jean-Pierre

Lefebvre:

ftp://ftp.mamalinux.com/pub/atftp/

Unfortunately, the Intel LANDesk Service Agent II prior to version

0.99h (PXE PDK V2.4) seems to have a rather serious bug: it requests

the TFTP "blksize" option, but will be mortally confused if this

option is actually accepted by the server!  There are three possible

workarounds for this bug:

1. Use a TFTP server with doesn't support "blksize".

Unfortunately, PXELINUX requires the "tsize" option to be

supported, and it is very unusual for TFTP servers to implement one

and not the other.  Both the tftp-hpa and atftp TFTP servers (see

above) therefore can be configured to disable the blksize option.

Use "-r blksize" for tftp-hpa, "--no-blksize" for atftp.

2. Use MTFTP for the initial bootstrap.  You need an MTFTP server with

the appropriate DHCP setup to do this.

3. Use appropriate DHCP options that the client will attempt MTFTP

before trying conventional TFTP.  It seems that the client will not

request the "blksize" option if it has tried MTFTP and failed.

These DHCP options are beyond the scope of this document.

++++ SETTING UP THE DHCP SERVER ++++

The PXE protocol uses a very complex set of extensions to DHCP or

BOOTP.  The best is probably to set up a "PXE boot server" on port

4011 of your TFTP server; a free PXE boot server is available at:

http://www.kano.org.uk/projects/pxe/

With such a boot server defined, you should be able to use the

following DHCP set up, using ISC dhcp 2.0 dhcpd.conf syntax:

allow booting;

allow bootp;

# PXE-specific configuration directives...

option dhcp-class-identifier "PXEClient";

next-server <pxe-boot-server>;

# Standard configuration directives...

option domain-name "<domain name>";

option subnet-mask <subnet mask>;

option broadcast-address <broadcast address>;

option domain-name-servers <dns servers>;

option routers <default router>;

# Skip everything below if you want to use dynamic IP

# address assignment

host <hostname> {

hardware ethernet <ethernet address>;

fixed-address <hostname>;

}

Some PXE implementations can be coaxed to operate with a normal DHCP

server; unfortunately so far no "magic bullet" which works with all

PXE implementations work.

The best one currently known, using ISC dhcp 2.0 dhcpd.conf syntax:

allow booting;

allow bootp;

# PXE-specific configuration directives...

filename "/tftpboot/pxelinux.0" ;

option dhcp-class-identifier "PXEClient";

option vendor-encapsulated-options 09:0f:80:00:0c:4e:65:74:77:6f:72:6b:20:62:6f:6f:74:0a:07:00:50:72:6f:6d:70:74:06:01:02:08:03:80:00:00:47:04:80:00:00:00:ff;

next-server <TFTP-server>;

# server-identifier <TFTP-server>;	# Illegal! See below

# Standard configuration directives...

# Same as above

However, some PXE implementations will only work this way if the

"dhcp-class-identifier" and "vendor-encapsulated-options" are *not*

present.  Intel Boot Client 3.0 and later are known to fall into this

category.  Arguably, that would have been the right thing from the

beginning.

Note that if your particular TFTP daemon runs under chroot (tftp-hpa

will do this if you specify the -s (secure) option; this is

recommended), you most likely should not include the /tftpboot prefix.

If the DHCP and TFTP servers are not on the same machine, you are

supposed to set up a PXE boot server (see above) on port 4011 on the

TFTP server; the "PXE boot server" is basically a modified DHCP server

on a nonstandard port.  If this is not possible, you can try adding

the directive "server-identifier <TFTP-server>;" Unfortunately this is

a violation of DHCP protocol, but some PXE implementations do not

accept the next-server directive without it or the "boot server".

I have successfully booted an Intel N440BX ("Nightshade") motherboard

with onboard Ethernet and "Intel LANDesk(R) Service Agent II version

0.99c" (derived from the Intel PXE PDK V2.0) and the Intel PRO/100+

Management Adapter "Boot agent 2.6 (build 071)" (derived from the

Intel PXE PDK V3.0 bld 071) using the configuration above and ISC

dhcpd 2.0.

++++ SOME NOTES ++++

If the boot fails, PXELINUX (unlike SYSLINUX) will not wait forever;

rather, if it has not received any input for approximately five

minutes after displaying an error message, it will reset the machine.

This allows an unattended machine to recover in case it had bad enough

luck of trying to boot at the same time the TFTP server goes down.

++++ PXELINUX IS STILL BETA ++++

PXELINUX is still very much beta; however, I'd be interested in

hearing about any experiences you might have with it, good or bad.  If

you have any comments, please use the SYSLINUX mailing list mentioned

at the end of syslinux.doc.  Thanks!

Currently known problems:

+ Requires a TFTP server which supports the "tsize" option.

+ The error recovery routine doesn't work quite right.  For right now,

it just does a hard reset - seems good enough.

+ There may be funnies with memory management.  The PXE spec has no

decent way of telling it to free up all memory and unchain any

interrupts; it allows the base stack to be unloaded, but not the

UNDI driver.

+ There seems to be a problem with sending ACK "storms"; a number of

ACK packets fired off without the proper delay in between.  I

suspect this is a PXE firmware problem, rather than a PXELINUX

problem.

+ We should probably call the UDP receive function in the keyboard

entry loop, so that we answer ARP requests.

+ Boot sectors don't work yet... they probably need auxilliary information

(such as device) to work at all.

* No way to chain-load onto a hard disk.